| Mark |  |
| --- | --- |

| Team name: | B2 |  |  |
| --- | --- | --- | --- |
| Homework number: | HOMEWORK 03 |  |  |
| Due date: | 11/10/2026 |  |  |
| Contribution | NO | Partial | Full |
| Boglioli Alessandro |  |  | x |
| Butti Stefano |  |  | x |
| Colombi Riccardo |  |  | x |
| Garatti Elia |  |  | x |
| Stilla Chiara |  |  | x |
| Notes: none |  |  |  |

| Project name | Play a song |  |  |
| --- | --- | --- | --- |
| Not done | Partially done (major problems) | Partially done (minor problems) | Completed |
|  |  |  | x |
| Explanation: We successfully completed the homework. Next, we explain all the steps for completing the homework: We connected pin PA9 to TIM1_CH2 and we set PA8 to GPIO_EXTI8. We used two timers: TIM1 is used to play the note on the buzzer and TIM2 is used to define the duration of the notes. For TIM1 we set channel 2 to PWM Generation CH2, clock source to Internal Clock, the prescaler to 1000-1 and pulse and counter period to the values for the first note of the melody. For TIM2 we also set clock source to Internal Clock, prescaler to 8400-1 and counter period to 2500-1. TIM2 will count the eighth notes, we decided to make them last 0,25 seconds. The song we decided to play is “London Bridge is Falling Down”. In the NVIC section we enabled “EXTI line[9:5] interrupts” and “TIM2 global interrupt”. In the GPIO section we set PA8 to “External Interrupt Mode with Rising edge trigger detection” and we generated the code. |  |  |  |
| We defined the following: - struct “note”: stores period and pulse of the notes. - “notes” array: array of all the notes of the melody in order - “notes_frequencies” array: contains notes frequencies, used to initialize “notes” array - “is_playing” variable: used to makes sure that the song finishes before starting again We defined a “initialized_notes()” method that takes the frequencies of the notes from “notes_frequencies” and converts them to the correct pulse and period values and stores the result in the “notes” array. The function is called at the start of the execution. We overwrote HAL_TIM_PeriodElapsedCallback: - We check if the function has been called by TIM2 - We move to the next note, and if the song is not finished yet we update the pulse and period values of TIM1 to the ones of the next note. If the next note has a shorter period with respect to the current counter, we first reset the counter - If the song has finished we stop the two timers and reset the “is_playing” variable We overwrote HAL_GPIO_EXTI_Callback: - We check if it has been called by pin 8 (the microphone) and if the song is not playing - We set the “is_playing” variable to 1 and reset the note index - We set the period and the pulse of TIM1 to the values of the first note - We reset the timer’s counters - We start the timers |  |  |  |
| Professor comments: |  |  |  |