| Mark |  |
| --- | --- |

| Team name: | B2 |  |  |
| --- | --- | --- | --- |
| Homework number: | HOMEWORK 02 |  |  |
| Due date: | 04/10/2026 |  |  |
| Contribution | NO | Partial | Full |
| Boglioli Alessandro |  |  | x |
| Butti Stefano |  |  | x |
| Colombi Riccardo |  |  | x |
| Garatti Elia |  |  | x |
| Stilla Chiara |  |  | x |
| Notes: none |  |  |  |

| Project name | Toggle the green LED on each finger snap |  |  |
| --- | --- | --- | --- |
| Not done | Partially done (major problems) | Partially done (minor problems) | Completed |
|  |  |  | x |
| Explanation: We successfully completed the homework. Next, we explain all the steps for completing the homework: We used “Hands-on lab schematic” and “NUCLEO schematic” in order to find the pins corresponding to the microphone (SND_IN) and the green led (LD2). Microphone pin: PA8 Green led pin: PA5 We created a project on STM32CubeMX configuring the PA8 pin as EXTI and the PA5 pin as Output. In NVIC section we selected “EXTI line[9:5] interrupts” with priority 0. In GPIO section we modified pin PA8 mode to “External Interrupt Mode with Rising edge trigger detection” and we generated the code. We overwrote the callback function, HAL_GPIO_EXTI_Callback, in main.c to handle the main logic behaviour, using the following HAL_Functions (taken from the HAL Functions document): - HAL_GPIO_ReadPin - HAL_GPIO_WritePin - HAL_GetTick |  |  |  |
| In the callback function: - We check that the callback is issued by pin 8; - We check that the previous callback was issued more than 50ms earlier (software debouncing); using the GetTick function we check how much time passed from the previous callback, if not enough time has passed, we ignore the signal and avoid executing the callback logic; - We read the current state of the led; - We change it to the opposite state. |  |  |  |
| Professor comments: |  |  |  |